Tender text

FlowBox HK20 pump group

WATTS FlowBox HK20 UPM3 A 15-70

Pump group for connecting boilers to unmixed heating circuits. Space-saving design, centre distance of 90 mm, pump length of 130 mm. All WATTS pump groups and boiler manifolds are of uniform design. A patented EPP insulation shell provides a highly effective heat insulation conforming to EnEV (German Energy Saving Ordinance). Simple installation. Concealed cable routing behind insulation. Includes an additional ball valve for the supply line below the pump. All connections G1" male thread, flat sealing. For operation in closed heating networks with non-aggressive media, such as water or water-glycol mixes as per VDI (Association of German Engineers) 2035/ÖNORM (Austrian standard) H 5195. Certified to in-house standard WI10077143N.

Consists of:

- Multi-function isolation valve for supply line with adjustable gravity brake, thermometer (0-120 °C), integrated temperature sensor connector and red handle.

- Multi-function isolation valve for return line with thermometer (0-120 °C), integrated temperature sensor connector and blue handle.

- Wall bracket

- Patented three-part EPP insulation shell

- High-efficiency pump

- Stainless steel pipe

- Ball valve for supply line below the pump

Ball valves: Brass body, spindle and coupling nut.

Technical data

Nominal size: DN20

Centre distance: 90 mm

Overall pump length: 130 mm

Height: 293 mm (sealing surface to sealing surface)

Height, including insulation: 310 mm

Width, including insulation: 232 mm

Depth, including insulation: 217 mm

Max. operating pressure: 6 bar
Max. operating temperature: +90° C

Recommended range of application
(at DT=20 K, Dp=100 mbar): max. 55 kW

Weight: 4.0 kg

Weight, including packaging: 4.4 kg

Pump: Grundfos UPM3 Auto L 15-70

Power supply: 1~230 V +10%/-15%, 50/60 Hz (acc. to IEC 60038)

Protection class: IP 44

Insulation class: F

Compliant with RoHS

Cable length: 2 m

Type HK20 UPM3 A 15-70

Part no.: 10077143

EAN: 4051394129606

Manufacturer: Watts Water Technologies

We reserve the right to make technical changes!

Proof of delivery:

Watts Industries Deutschland GmbH

Godramsteiner Hauptstr. 167

76829 Landau

Germany

Phone +49 63 41 96 56-0

Fax +49 63 41 96 56-560

WIDE@wattswater.com

www.wattswater.de