ULTRAMIX OMDA thermostatic mixing valve with interchangeable cartridge

Thermostatic mixing valve for hydrotherapy, balneotherapy, or medical applications. Specifically conceived to withstand seawater, softened water and distilled water.

Maxi pressure: 10 bar.

Mini pressure: 1 bar.

Recommended pressure: 2-4 bar.

Maximum hot water temperature: 85°C.

ACS approval.

Degree graduation, adjustment range for 10 to 50°C (or others to specify), easily calibrated to the system's hydraulic conditions.

Mechanism easily dismountable and equipped with screws, jets of diaphragm and hoppers made in stainless steel.

Dismountable thermostatic mechanism equipped with protection filters and check valves.

Setpoint temperature not sensitive to flow rate variations in the installation, whether at minimum or maximum.

Adjustable and lockable maximum temperature limit.

Bimetallic strip technology.

Easy and fast maintenance.

Precise control and comfortable temperature stability at both low and high flow rates.

Anti-scalding security: automatic shut-off in the event of cold-water supply failure.

Answer against legionella.